Honda Diagnostic Trouble Codes (DTC)

| Code | Accord | Civic | CRX | Prelude | Error |
| --- | --- | --- | --- | --- | --- |
| 1 | 85-98 | 86-98 | 85-91 | 85-98 | Oxygen Sensor "A" (Primary) defective circuit or unplugged / defective sensor |
| 2 | 85-89 | 86-87 | 85-87 | NA | Oxygen Sensor "B" defective circuit or unplugged / defective sensor |
| 3 | 85-98 | 86-98 | 85-91 | 85-98 | MAP Sensor (Manifold Absolute Pressure) defective circuit or unplugged / defective sensor |
| 4 | 90-98 | 88-98 | 88-91 | 85-98 | CKP Sensor (Crankshaft Position Sensor) defective circuit or unplugged / defective sensor |
| 5 | 85-98* | 86-98 | 85-91 | 85-98 | MAP Sensor (Manifold Absolute Pressure) mechanical problem / disconnected piping * exc 94-95 |
| 6 | 85-98 | 86-98 | 85-91 | 85-98 | ECT Sensor (Engine Coolant Temperature) defective circuit or unplugged / defective sensor |
| 7 | 85-98 | 86-98 | 85-91 | 85-98 | TP Sensor (Throttle Position) defective circuit or unplugged / defective sensor |
| 8 | 85-98 | 86-98 | 85-91 | 85-98 | TDC Sensor (Top Dead Center) defective circuit or unplugged / defective sensor |
| 9* | 85-98** | 86-98 | 85-91 | 85-98 | CYP Sensor (Cylinder) defective circuit or unplugged / defective sensor * not a valid code for any throttle body injection Hondas ** not a valid code for V-6 engines |
| 10 | 85-98 | 86-98 | 85-91 | 85-98 | IAT Sensor (Intake Air Temperature) defective circuit or unplugged / defective sensor |
| 12 | 85-98 | 86-98* | 85-91* | 85-98 | EGR Lift Sensor (Exhaust Gas Recirculation) defective circuit or unplugged / defective sensor * not a valid code if car is not equipped with an EGR valve (some Civic models are not) |
| 13 | 85-98 | 86-98 | 85-91 | 85-98 | BARO Sensor (Atmospheric Pressure) defective circuit or unplugged / defective sensor |
| 14 | 88-98 | 88-98 | 88-91 | 88-98 | IAC Valve (Idle Air Control) defective circuit or unplugged / defective sensor |
| 15 | 90-95 | 88-95 | 88-91 | 88-95 | Ignition Output Signal missing or defective ignition output signal |
| 16 | 90-95 | 88-95 | 88-91 | 88-95 | Fuel Injector System defective circuit or unplugged / defective fuel injector |
| 17 | 88-98 | 88-98 | 88-91 | 88-98 | VSS (Vehicle Speed Sensor) defective circuit or unplugged / defective sensor |
| 19 | NA | 88-95 | 88-91 | NA | Automatic Transmission Lock Up Control Solenoid Valve defective circuit or unplugged / defective solenoid valve |
| 20 | 90-98 | 88-98 | 88-91 | 92-98 | Electrical Load Detector defective circuit or unplugged / defective sensor |
| 21* | 94-98** | 92-95 | NA | 93-95 | VTEC Solenoid Valve defective circuit or unplugged / defective solenoid valve * not a valid code if car does not have a VTEC engine ** not with V-6 engine |
| 22* | 94-98 | 92-98 | NA | 92-98 | VTEC Oil Pressure Switch defective circuit or unplugged / defective oil pressure switch * not a valid code if car does not have a VTEC engine |
| 23 | 98* | 96-98 | NA | 92-98 | KS (Knock Sensor) defective circuit or unplugged / defective sensor * not a valid code if V-6 engine |
| 30 | 90-98* | NA | NA | 92-98* | Automatic Transmission Signal: "A" / SEAF / SEFA / TMA or TMB defective circuit or unplugged / defective sensor * not a valid code for 1996 |
| 31 | 90-95 | NA | NA | 92-95 | Automatic Transmission Signal "B" defective circuit or unplugged / defective sensor |
| 41 | 90-98 | 92-98 | NA | 90-98 | Primary Oxygen Sensor - Heater circuit malfunction |
| 43 | 90-95 | 92-95 | NA | 90-95 | Fuel Supply System defective or malfunctioning fuel supply system |
| 45 | 96-98 | 96-98 | NA | 96-98 | System Too Lean or Too Rich malfunction in the fuel monitoring systems |
| 48 | 98 | 92-98 | NA | NA | LAF Sensor (Lean Air Fuel) defective circuit or unplugged / defective sensor |
| 54 | 96 | 96-98 | NA | NA | CKF Sensor (Crankshaft Speed Fluctuation) defective circuit or unplugged / defective sensor |
| 58 | 98* | NA | NA | NA | TDC Sensor 2 (Top Dead Center) defective circuit or unplugged / defective sensor * with V-6 engine only |
| 61 | 96-98 | 96-98 | NA | 96-98 | Oxygen Sensor, Heated - Sensor 1 (Primary) high voltage, low voltage, or slow response |
| 63 | 96-98 | 96-98 | NA | 96-98 | Oxygen Sensor, Heated - Sensor 2 (Secondary) high voltage, low voltage, or slow response |
| 65 | 96-98 | 96-98 | NA | 96-98 | Oxygen Sensor Heater (Secondary) malfunctioning or defective oxygen sensor heater |
| 67 | 96-98 | 96-98 | NA | 96-98 | Catalyst system Efficiency Below Threshold malfunctioning or defective catalyst system |
| 70 | 96-98 | 96-98 | NA | 96-98 | Automatic Transmission malfunction with the automatic transmission controls |
| 71 | 96-98 | 96-98 | NA | 96-98 | Cylinder 1 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire |
| 72 | 96-98 | 96-98 | NA | 96-98 | Cylinder 2 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire |
| 73 | 96-98 | 96-98 | NA | 96-98 | Cylinder 3 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire |
| 74 | 96-98 | 96-98 | NA | 96-98 | Cylinder 4 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire |
| 75 | 98* | NA | NA | NA | Cylinder 5 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire * valid code for V-6 engine only |
| 76 | 98* | NA | NA | NA | Cylinder 6 Misfire or a Random Misfire a condition is present that is creating a cylinder misfire * valid code for V-6 engine only |
| 80 | 96-98 | 96-98 | NA | 96-98 | Exhaust Gas Recirculation insufficient flow detected |
| 86 | 96-98 | 96-98 | NA | 96-98 | ECT Sensor (Engine Coolant Temperature) circuit range / performance problem |
| 90 | 97-98 | 96-98 | NA | 97-98 | Evaporative Emission Control System leak detected in the fuel tank area |
| 91 | 96-98 | 96-98 | NA | 97-98 | Fuel Tank Pressure Sensor low input |
| 92 | 96-98 | 96-98 | NA | 96-98 | Evaporative Emission Control System insufficient purge flow |

Code 0 _____ Faulty ECU

Code 1 _____ Oxygen sensor A (defective circuit or unplugged / defective sensor)

Code 2 _____ Oxygen sensor B (defective circuit or unplugged / defective sensor)

Code 3 _____ MAP sensor - Manifold Absolute Pressure (defecitve circuit or unplugged / defective sensor)

Code 4 _____ CKP Sensor - Crank Position Sensor (defective circuit or unplugged / defective sensor)

Code 5 _____ MAP sensor again (mechanical problem / disconnected piping)

Code 6 _____ Engine Coolant temperature (defective circuit or unplugged / defective sensor)

Code 7 _____ TP Sensor - Throttle Position (defective circuit or unplugged / defective sensor)

Code 8 _____ Top Dead Center Sensor (defective circuit or unplugged / defective sensor)

Code 9 _____ No. 1 Cylinder Position Sensor (only on 1.6L) (defective circuit or unplugged / defective sensor)

Code 10 ____ Intake Air Temperature Sensor (defective circuit or unplugged / defective sensor)

Code 11 ____ Does not appear to exist

Code 12 ____ Exhaust Gas Recirculation Lift Sensor (defective circuit or unplugged / defective sensor)

Code 13 ____ Atmospheric Pressure (defective circuit or unplugged / defective sensor)

Code 14 ____ Idle Air Control (IACValve) (defective circuit or unplugged / defective sensor)

Code 15 ____ Ignition Output Signal (missing or defective output signal)

Code 16 ____ Fuel Injector System (defective circuit or unplugged / defective sensor)

Code 17 ____ Vehicle Speed Sensor (defective circuit or unplugged / defective sensor)

Code 18 ____ Does not appear to exist

Code 19 ____ Lock up control solenoid (auto trans only)

Code 20 ____ Electric Load Detector (defective circuit or unplugged / defective sensor)

Code 21 ____ VTEC spool solenoid valve

Code 22 ____ VTEC pressure valve

Code 23 ____ Knock sensor

Code 30 ____ Automatic transmission A signal

Code 31 ____ Automatic transmission B signal

Code 36 ____ traction control found on JDM ECUs

Code 41 ____ Primary oxygen sensor heater

Code 43 ____ Fuel supply system

Code 45 ____ Fuel system too rich or lean

Code 48 ____ LAF - lean air fuel sensor

Code 54 ____ CKF - crank fluctuation sensor

Code 58 ____ TDC sensor #2

Code 61 ____ Primary oxygen sensor

Code 63 ____ Secondary oxygen sensor

Code 65 ____ Secondary oxygen sensor heater

Code 71 ____ random misfire cylinder 1

Code 72 ____ random misfire cylinder 2

Code 73 ____ random misfire cylinder 3

Code 74 ____ random misfire cylinder 4

Code 80 ____ Exhaust Gas Recirculation insufficient flow detected

Code 86 ____ ECT Sensor (Engine Coolant Temperature) circuit range / performance problem

Code 90 ____ Evaporative Emission Control System leak detected in the fuel tank area

Code 91 ____ Evaporative Emission Control System insufficient purge flow

0 Faulty ECU (no Signal) 
1 Oxygen sensor or circuit 
2 Faulty ECU 
3 Manifold Absolute Pressure (MAP) sensor or circuit. 
4 Crank angle sensor or circuit 
5 Manifold Absolute Pressure (MAP) sensor or circuit. 
6 Coolant temperature sensor or circuit 
7 Throttle angle sensor or circuit. 
8 TDC position/crank angle sensor or circuit. 
9 Crank angle sensor or circuit 
10 Intake air temperature sensor or circuit. 
11 No particular symptom shown or system does not operate -faulty ECU 
12 Exhaust gas recirculation (EGR) failure 
13 Atmospheric pressure sensor circuit. 
14 Electronic air control valve (EACV) 
15 No ignition output signal -possible faulty igniter. 
16 Fuel injector circuit. 
17 Vehicle speed sensor or circuit (VSS) 
19 Lock-up control solenoid valve (automatic transmission) 
20 Electronic load detector -possible open or grounded circuit in ECU wiring. 
21 VTEC spool solenoid valve circuit (Civic & DeI-Sol) 
22 VTEC oil pressure switch circuit (Civic & Del-Sol) 
23 Knock sensor- (prelude) 
30 NT control unit ECM fuel injection signal "A" (Accord & Prelude) 
31 NT control unit ECM fuel injection signal "B" (Accord & Prelude) 
41 Heated oxygen sensor -heater circuit. 
43 Fuel supply system circuit (except Dl5Z1 engine) 
43 xx 
45 Fuel system out of range -rich or lean 
48 Heated oxygen sensor circuit (D15Z1 engine) 
50 Mass airflow sensor. 
61 Front oxygen sensor response slow. 
63 Rear oxygen sensor circuit voltage out of range. 
65 Rear oxygen sensor circuit malfunction. 
67 Catalytic converter efficiency low. 
70 Automatic Transaxle problem 
71 Trough 76 Cylinder misfires 
80 Insufficient EGR flow . 
86 Engine coolant temperature sensor circuit. 
92 Evaporative emission purge valve problem.